# Door Monitor — User Guide

A two-piece wireless door monitor. Unit 1 (Sensor) sits by the door and senses open or closed. Unit 2 (Display) shows the door’s status on a round screen, anywhere within wireless range. The two units talk directly to each other — no Wi-Fi network, app, or account is needed.

## Turning it on

- Unit 1 (Sensor): plug its USB port into any 5 V USB power source

- Unit 2 (Display): plug its USB-C port into a 5 V USB power source

- Pairing: The units pair automatically within a second or two after both are powered on when in range. No setup is required. If no paring units are too far apart.

## Door Unit 1 (Sensor) — the three lights

| Light | What it means |
| --- | --- |
| Red — Power | On whenever the unit has power. This is just the power indicator (always on). |
| Comm | Link to the display: red = not connected, green = connected, brief blue flash = message sent/received. Normal at start: red for about a second, then green. |
| Green — Door | On = door CLOSED. Off = door OPEN. |

A healthy power-up: the red power light comes on, the Comm light flashes red then settles on green once it finds the display, and the green Door light matches your door.

## Unit 2 (Display) — the screen

| Screen | What it means |
| --- | --- |
| PAIRING… | Starting up / looking for the sensor (first couple of seconds). |
| Red — OPEN | The door is open. |
| Dimmed — CLOSED | The door is closed. A small green dot blinks with each update, so you can see it is live. |
| Amber — NO SIGNAL | Lost contact with the sensor — sensor unpowered, out of range, or restarting. The edge of the screen blinks to get your attention. |

## Known issue

Known issue: Unit 1 sometimes doesn’t start (only the red light) Once in a while at power-up, only the red power light comes on — the Comm light and the green Door light stay dark — and the unit does nothing. This is a start-up glitch: a brief voltage dip as it powers on stops it from booting. Fix: unplug Unit 1, wait a couple of seconds, and plug it back in. It will start normally. How to tell it apart: a good start lights the Comm light (red → green) and the green Door light within a second or two. If only the red power light stays lit, do the unplug / re-plug.

## Quick help

- Display shows “NO SIGNAL”: the sensor isn’t reaching it. Check Unit 1 is powered and actually started (see Known issue), and within range — about 10–30 m indoors through walls, 100–200 m line-of-sight.

- It seems backwards (says CLOSED when open, or the green light is inverted): the magnet/switch alignment or a setting is reversed — let me know and I’ll flip it.